| Digitalization Industrialization Standardization Collaboration DISC DEXPI Equinor – Aker BP – Aker Solutions - Aibel |  |  |
| --- | --- | --- |
| Document title: |  |  |
| DISC DEXPI Symbol Definition Requirements |  |  |
| Document no.: | Rev.: | Page: |
|  | 0.1 | 1 of 18 |

DISC DEXPI Symbol Definition Requirements

Prepared for
DISC DEXPI Group

Prepared by

Pedersen, Tonia

Revision and Signoff Sheet

# Change Record

| Revision | Date | Reason for issue, key changes and or decisions | Prepared |
| --- | --- | --- | --- |

# Table of Contents

Change Record	3

Table of Contents	4

Overview	5

Purpose	5

Abbreviations	5

Process and Instrumentation Diagram Transfer	6

Overview	6

DrawIO symbol definition and svg export plugin	6

Overview	6

DrawIO symbol definition	6

Layer definition	6

‘Special’ symbol definition	6

Property definition	7

Dimension:	7

Connection Point:	7

Example file:	8

SVG Plug-in	8

Install	8

Configuration Options	8

Transfer Requirements	8

Symbols	10

ANNEX D: Symbol Rotation and mirroring - example	11

Rotation and Mirroring in DEXPI and Proteus	12

DEXPI	12

Proteus	13

ANNEX E: Signal conveying Line Types	13

ANNEX F: Valve Label Details	14

ANNEX G: Proteus Connection Points & Associations	15

Piping Connector	15

Instrument Connector	17

# Overview

## Purpose

This document describes the requirements for DISC DEXPI Profile Symbols as used for the creation of symbols is the engineering tools used to the create the P&ID and resultant DEXPI file and for verification of of these files.

## Abbreviations

|  | Description | Comment |
| --- | --- | --- |
| DISC | Digitalization Industrialization Standardization Collaboration |  |
| DEXPI | Data Exchange in the Process Industry |  |
| P&ID | Process & Instrument Diagram |  |
| OPC | Off Page Connector |  |
| PIF | ProcessInstrumentFunctions |  |
| DEXPI XML | Working name for DEXPI 2.0 serialization format |  |

# Process and Instrumentation Diagram Transfer

## Overview

The P&ID is a detailed graphical representation of the process flow and interconnection of the involved components, including piping, equipment, valves, instrumentation, and other process components.

Within the DISC DEXPI project the content of the P&ID has been transcribed into a digital information model DEXPI through the detailed use of symbol definitions and their mapping, either directly or through the use of project criteria to DEXPI class types or subtypes.

# DISC DEXPI Profile Symbol Definition

# Overview

A Profile/Symbol defines:

- The graphical appearance of a symbol

- Its symbol variants and variant-selection rules

- The node positions allowed for connections

- The label templates and their placement

- The rules for rotation and mirroring

- The mapping between symbols and DEXPI class elements

These requirements are used when validating a DEXPI file to ensure that:

1. The instantiated symbol matches the intended class element.

2. The symbol variant selected in the drawing corresponds to the attribute/value condition in the DEXPI data.

3. The node connections comply with the declared node position types and min/max constraints.

4. The labels shown in the drawing follow the specified positions and attribute references.

# Symbol Definition and Identity Requirements

The symbol definition applied within the DISC DEXPI project provides the details needed to ensure that the CAD vendors can create each symbol using defined dimensions along with specified origin and connection point ordinates.

- 2.1 Unique Symbol Reference

Each symbol must define a unique:

Symbol name (Object/@name at Profile/Symbol level)

SymbolVariant name (Object/@name under Variants)

These must be unique within the profile namespace and serve as the reference for validation.

- 2.2 MetaData Usage Requirement

A symbol must specify the intended DEXPI class element through the property Data property="MetaData/usage". Validation uses this to ensure that a P&ID symbol instance is linked to a DEXPI element of a compatible class.

- 3. Origo and Coordinate System Requirements

- 3.1 Symbol Origo

The origo is defined as the (0,0) coordinate of the symbol coordinate system. All graphical primitives, node positions, and label positions must be defined relative to (0,0). Validation verifies that the instantiated symbol is geometrically consistent when translated/scaled into drawing coordinates.

- 3.2 Bounding Box

Each variant must declare MinX, MaxX, MinY, and MaxY. These values define the graphical bounds for collision detection and layout verification.

- 4. Symbol Variant Requirements

- 4.1 Base Variant

At least one variant must be unconditional (no Condition property). This is the default representation.

- 4.2 Conditional Variants

Each conditional symbol variant must declare a Condition with an attribute/value pair, e.g.:
ValvePosition = '1NCStraight'

Validation must ensure that the associated DEXPI class element contains the attribute with the matching value. Variant numbers (VariantNumber) must be unique and used for ordering but are not used in validation.

- 4.3 Variant Selection Logic

During validation:
1. The DEXPI element’s attributes are evaluated.
2. The first variant whose Condition evaluates to true is selected.
3. If no conditioned variant applies, the base variant is used.
4. The DEXPI file’s chosen SymbolVariant must match the symbol reference stored in the drawing.

Failure to match produces a variant mismatch validation error.

- 5. Graphical Representation Requirements

- 5.1 Primitive Elements

Symbols must be composed of graphical primitives such as:

Core/Diagram.Polygon

Core/Diagram.Line

Core/Diagram.Ellipse

Core/Diagram.Stroke

Core/Diagram.FillStyle

Validation checks that all required primitives are present for the declared variant, that the coordinates match the expected geometry, and that stroke and fill styles meet the reference definition (within a tolerance for rendering differences).

- 5.2 Fill and Stroke Rules

Primitives may be transparent or filled. Stroke width, color, and dashstyle must match the definition (within a reasonable tolerance).

- 6. NodePosition Requirements

A symbol variant must provide one or more NodePositions, defining:

Position (X,Y)

Type (e.g., Profile/NodePositionType.Piping)

MinCount

MaxCount

Direction (for auto-routing only; not used during validation)

- 6.1 Validation Requirements for Connections

During DEXPI verification:
1. Each connected line must correspond to the declared node type (e.g., a piping node only allows Piping connections).
2. The number of connections must satisfy: MinCount ≤ actualConnections ≤ MaxCount.
3. The drawing-selected node must align geometrically with the NodePosition coordinates (within a tolerance).

- 6.2 Node Direction

Directions (e.g., 180°, 0°, 270°) assist automated drawing generation. Validation does not require direction checking.

Figure 1: Connection line angle intercept value reference

- 7. Label Template Requirements

A symbol variant may define one or more LabelTemplates describing:

Alignment (Top, Bottom, Center, etc.)

Font and Size

Color

Label Position (X,Y) relative to origo

Rotation (usually 0° unless explicitly rotated)

Text containing attribute placeholders (e.g., <ObjectDisplayName>, <NominalDiameter>)

- 7.1 Validation Requirements

For each label:
1. Placement: The label must appear near the expected coordinates.
2. Attributes: All attribute placeholders must correspond to attributes available on the associated DEXPI class element.
3. Formatting: Ordering and grouping of attributes must follow the LabelTemplate specification.

Label content may be empty if the corresponding attribute is empty in the DEXPI file.

- 8. Rotation and Mirroring Requirements

Symbols specify RotationAllowed (0 = allowed, 1 = not allowed) and MirroringAllowed (0 = allowed, 1 = not allowed). Validation checks that a symbol instance is not rotated if rotation is prohibited and not mirrored if mirroring is prohibited. If the drawing uses a rotation or mirror that contradicts the allowed values, this produces a geometry violation.

- 9. Mapping Symbol to Class Element(s)

Each symbol must be mapped to one or more DEXPI classes via the MetaData/usage property, for example:
DiscProfile.InformationModel.ThreeWayValve

Validation rules:
1. The symbol instance must reference a DEXPI element whose class is compatible with the declared usage.
2. If a symbol supports multiple classes, the variant selection logic must remain valid across all supported classes.
3. Attribute/value pairs used in variant conditions must belong to the referenced class.

- 10. Summary of Validation Checks

The validator shall confirm all of the following:

Symbol identity: symbol name and class usage match the profile definition.

Geometry: variant exists, selected variant matches attribute logic, primitives match, no illegal rotation/mirroring.

Node connections: each connection aligns to a NodePosition, types match, and Min/MaxCount is respected.

Labels: label placement, attribute placeholders, and text expansion are correct.

Variant condition: DEXPI attributes match Condition rules or base variant applies.

- 11. Conclusion

This requirements document establishes a complete verification framework for DISC DEXPI Profile Symbols. It ensures consistency between the P&ID graphical representation, the selected symbol variant, the DEXPI class element data, the incoming and outgoing connections, and the label content and positioning. Following these requirements guarantees accurate, standard

Figure 2: Symbol Rotation Attribute Example

# ANNEX E: Signal conveying Line Types

Refer to DISC DEXPI Symbol legend file Ref: Error! Reference source not found.

# ANNEX F: Valve Label Details

The attributes available within valve labels shall be determined based on whether the valve symbol is used to show a manual valve or an on/off actuated or controlled valve. Attribute details for the valve label is defined in the symbol legend details (Ref: Error! Reference source not found.) using the attribute formats below.

- Manual Valves

A space character shall be used between the trim and lock mechanism attributes as shown.

<ObjectDisplayName>

<NominalDiameter><VDS>

<TrimType> <LockMechanism>

- On/Off Actuated or Controlled Valves

A space character shall be used between the trim and lock mechanism attributes as shown.

<NominalDiameter><VDS>

<TrimType> <LockMechanism>

Additional Notes:

- Valve Normal Position is no longer required to be shown in the label. Normal Position will be indicated by the symbol used e.g. black  closed , white  open

- Failure action is required to be shown in the label for the actuator if available Ref: Error! Reference source not found.

- “Barrier information is not required in the label as this will be shown as part of the graphic” referring to DIB valves (examples below) and “Barrier information” the DP and SR notations. The following DIB valves will be added to the DISC DEXPI symbol legend soon.

Figure 3: Valve Barrier variants